Journal of Geotechnical and Geoenvironmental Engineering

Supplemental Materials for

Physics-Guided Transfer Learning of Soil Freezing Characteristic Parameterization for Coupled Thermo-Hydro-Chemical Simulation of Saline Permafrost

Xueyang Wang, S.M. ASCE1, Ming Xiao, Ph.D., P.E., F.ASCE2

1Graduate student, Department of Civil and Environmental Engineering, The Pennsylvania State University, University Park, PA 16802, United States of America. Email: xxw5360@psu.edu (Corresponding author)

2 Professor, Department of Civil and Environmental Engineering, The Pennsylvania State University, University Park, PA 16802, United States of America. Email: mzx102@psu.edu

Introduction

The supplemental material includes supporting information for the Physics-guided Transfer learning (PG-TL) framework:

- Data Sources for SFC Database Compilation

- Missing-metadata Recovery and Imputation

- Neural Network Architecture and Training Hyperparameters

# Text. S1  Data Sources for SFC Database Compilation

The 45 publications used to compile the SFC samples are summarized by reported sample locations to document data provenance and traceability (Table S1).

Table S1. Data sources in the literature grouped by sample locations

| Country | Reference |
| --- | --- |
| Canada | Aksenov et al. n.d.; Azmatch et al. 2012; Elkhoraibi 1975; Hivon and Sego 1995; Patterson and Smith 1985; Penner 1970; Ren and Vanapalli 2019, 2020; Smerdon and Mendoza 2010; Smith and Tice 1988; Williams 1964 |
| China | Bi et al. 2020; Chai et al. 2018; Chen et al. 2024; Kong et al. 2020; Li et al. 2022, 2020; Liu et al. 2024, 2020; Ma et al. 2023; Meng et al. 2020; Ming et al. 2020; Tang et al. 2018, 2020; Teng et al. 2020; Wang et al. 2021; Wen et al. 2012; Wu et al. 2021, 2015; Xin et al. 2023; Zhang et al. 2019 |
| Japan | Suzuki 2004; Watanabe and Mizoguchi 2002; Watanabe and Osada 2017; Watanabe and Wake 2009 |
| Korea | Christ et al. 2009 |
| Russia | Chuvilin et al. 2022 |
| United States | Black and Tice 1989; Darrow et al. 2009; Dillon and Andersland 1966; Koopmans and Miller 1966; Kruse and Darrow 2017; Li 2009; Nakano et al. 1982; Smith and Tice 1988; Tice et al. 1988; Wang et al. 2026; Xiaozu et al. n.d.; Yoshikawa and Overduin 2005 |

# Text. S2  Missing-metadata Recovery and Imputation

Published investigations of SFC observations may only report a subset of the soil metadata needed for model development. To retain as many compiled samples as possible, missing metadata were completed in two steps. First, values that could be derived from physical relationships were recovered deterministically. Second, the remaining missing properties were estimated using statistical imputation.

In the first stage, variables in the phase-relation group (Gs, φ, Sr, ρd, and θtot) were recovered from any reported subset using standard phase-relation identities such as φ=1-ρd/(Gsρl). For samples explicitly described as non-organic and non-saline, SOM and Ms were set to zero. For samples whose SFC measurements extended to temperatures approaching the eutectic point but for which Sa was not reported, Sa was estimated from the residual gravimetric unfrozen water content near eutectic point, wb*, following Jin et al. (2020):

| Sa=wb*δ∙ρbw×10-2 | Eq. (1) |
| --- | --- |

where δ=5Å is the adsorbed-layer thickness and ρbw=0.985 g/cm³ is the density of strongly bound water. This estimation is physically motivated by the assumption that, at sufficiently low temperatures, the remaining unfrozen water is dominated by surface-adsorbed films whose mass scales linearly with mineral surface area.

In the second stage, remaining missing metadata were imputed using Generative Adversarial Imputation Nets (GAIN) (Yoon et al. 2018). The choice of GAIN is supported by interdependence among soil index properties. Previous studies have demonstrated significant correlations among Sa, clay content, PL, and LL in fine-grained soils (Petersen et al. 1996; Yukselen-Aksoy and Kaya 2010), and PTF have been developed to estimate PL and LL from soil texture and organic matter content (Keller and Dexter 2012). The GAIN model was trained on all thirteen soil index properties and returned a complete metadata matrix for all thirteen properties. Of the eleven properties used as inputs to the PG-TL model, eight (sand content, silt content, clay content, PL, LL, Sa, ρd, and φ) had missing values across all the soil samples. As shown in Figure S1, the missing-data rate of these eight properties ranged from 0.3% (φ) to 59.0% (Sa); masked-cell validation gave RMSE = 0.146, MAE = 0.083, and R² = 0.71.

Figure S1  Missing-data structure, validation accuracy, and distributional diagnostics of GAIN-based imputation

# Text. S3  Neural Network Architecture and Training Hyperparameters

A.1 Backbone architecture (SWCC pre-training)

The backbone fθ* is a fully connected multilayer perceptron with two hidden layers and LeakyReLU activation (Maas et al. 2013). Batch normalization is applied before the first activation to stabilize training across soils, and dropout (Srivastava et al. 2014) is applied after each activation for regularization. Four parallel linear heads map the hidden representation to the four VG parameters α*n*θr*θs*. The complete list of training hyperparameters is given in Table S2.

A.2 Residual adapter architecture (SFCC fine-tuning)

The adapter gϕ:Rds+dp→RHaconsists of 3 hidden layers with hidden dimension Ha=512, batch normalization, LeakyReLU activation, and dropout. During SFCC fine-tuning, the backbone was updated with a lower learning rate than the adapter and output heads, which retained the SWCC-trained pore-structure representation while allowing the residual adapter to learn freezing-specific corrections.

A.3 Hyperparameter optimization

Each trial evaluated a candidate configuration using five-fold cross-validation grouped by soil sample (Fushiki 2011) to prevent measurements from the same sample being split across training and validation folds. The optimized hyperparameters included component-wise learning rates (backbone, adapter, heads), dropout rates, weight decay, and the log-loss weight λlog. The optimized values are listed in Table S2.

A.4 GAIN imputation model

The GAIN model (Yoon et al. 2018) was implemented in PyTorch and trained for 5000 epochs with a hidden dimension of 96, batch size of 128, hint rate of 0.946, and reconstruction weight of 10. Masked-cell validation was performed by randomly masking known values, predicting them with the trained GAIN model, and comparing predictions to the original observations.

Table S2 Neural network architecture and training hyperparameters.

| Hyperparameters | Values |
| --- | --- |
| Pre-training (SWC) |  |
| Hidden dimension | 512 |
| Hidden layers | 2 |
| Activation | LeakyReLU (slope 0.1) |
| Normalization | BatchNorm |
| Dropout rate | 0.148 |
| Optimizer | Adam |
| Learning rate | 2.8 × 10⁻⁴ |
| Loss function | λlog = 0.126 |
| Fine-tuning (SFC) |  |
| Adapter dimension | 512 |
| Adapter layers | 3 |
| Activation | LeakyReLU (slope 0.1) |
| Dropout rate | 0.008 |
| Batch size | 16 |
| Backbone learning rate | 4.98 × 10⁻⁵ |
| Adapter learning rate | 2.66 × 10⁻⁴ |
| Head learning rate | 1.67 × 10⁻⁴ |
| Loss function | λlog = 0.261 |

References

Aksenov, V. I., G. I. Klinova, and I. V. Scheikin. 1998. “Material composition and strength characteristics of saline frozen soils.” In: 7Th Intl Permafr. Conf.

Azmatch, T. F., D. C. Sego, L. U. Arenson, and K. W. Biggar. 2012. “New ice lens initiation condition for frost heave in fine-grained soils.” Cold Regions Science and Technology, 82: 8–13. https://doi.org/10.1016/j.coldregions.2012.05.003.

Bi, J., M. Zhang, Y. Lai, W. Pei, J. Lu, Z. You, and D. Li. 2020. “A generalized model for calculating the thermal conductivity of freezing soils based on soil components and frost heave.” International Journal of Heat and Mass Transfer, 150: 119166. https://doi.org/10.1016/j.ijheatmasstransfer.2019.119166.

Black, P. B., and A. R. Tice. 1989. “Comparison of soil freezing curve and soil water curve data for Windsor sandy loam.” Water Resources Research, 25 (10): 2205–2210. https://doi.org/10.1029/WR025i010p02205.

Chai, M., J. Zhang, H. Zhang, Y. Mu, G. Sun, and Z. Yin. 2018. “A method for calculating unfrozen water content of silty clay with consideration of freezing point.” Applied Clay Science, 161: 474–481. https://doi.org/10.1016/j.clay.2018.05.015.

Chen, C., Z. Yang, Y. Deng, H. Ma, and J. Qian. 2024. “Experimental investigation of thawing behavior of saline soils using resistivity method.” Journal of Geophysics and Engineering, 21 (3): 854–867. https://doi.org/10.1093/jge/gxae037.

Christ, M., Y. C. Kim, and J. B. Park. 2009. “The influence of temperature and cycles on acoustic and mechanical properties of frozen soils.” KSCE Journal of Civil Engineering, 13 (3): 153–159. https://doi.org/10.1007/s12205-009-0153-1.

Chuvilin, E. M., B. A. Bukhanov, A. Z. Mukhametdinova, E. S. Grechishcheva, N. S. Sokolova, A. G. Alekseev, and V. A. Istomin. 2022. “Freezing point and unfrozen water contents of permafrost soils: Estimation by the water potential method.” Cold Regions Science and Technology, 196: 103488. https://doi.org/10.1016/j.coldregions.2022.103488.

Darrow, M. M., S. L. Huang, and S. Akagawa. 2009. “Adsorbed cation effects on the frost susceptibility of natural soils.” Cold Regions Science and Technology, 55 (3): 263–277. https://doi.org/10.1016/j.coldregions.2008.08.002.

Dillon, H. B., and O. B. Andersland. 1966. “Predicting unfrozen water contents in frozen soils.” Canadian Geotechnical Journal, 3 (2): 53–60. NRC Research Press. https://doi.org/10.1139/t66-007.

Elkhoraibi, M. C. E. 1975. “Volume change of frozen soils.” Carleton University. https://doi.org/10.22215/etd/1975-00289.

Hivon, E. G., and D. C. Sego. 1995. “Strength of frozen saline soils.” Canadian Geotechnical Journal, 32 (2): 336–354. https://doi.org/10.1139/t95-034.

Jin, X., W. Yang, X. Gao, J.-Q. Zhao, Z. Li, and J. Jiang. 2020. “Modeling the unfrozen water content of frozen soil based on the absorption effects of clay surfaces.” Water Resources Research, 56 (12): e2020WR027482. https://doi.org/10.1029/2020WR027482.

Keller, T., and A. R. Dexter. 2012. “Plastic limits of agricultural soils as functions of soil texture and organic matter content.” Soil Research, 50 (1): 7–17. https://doi.org/10.1071/SR11174.

Kong, L., Y. Wang, W. Sun, and J. Qi. 2020. “Influence of plasticity on unfrozen water content of frozen soils as determined by nuclear magnetic resonance.” Cold Regions Science and Technology, 172: 102993. https://doi.org/10.1016/j.coldregions.2020.102993.

Koopmans, R. W. R., and R. D. Miller. 1966. “Soil freezing and soil water characteristic curves.” Soil Science Society of America Journal, 30 (6): 680–685. https://doi.org/10.2136/sssaj1966.03615995003000060011x.

Kruse, A. M., and M. M. Darrow. 2017. “Adsorbed cation effects on unfrozen water in fine-grained frozen soil measured using pulsed nuclear magnetic resonance.” Cold Regions Science and Technology, 142: 42–54. https://doi.org/10.1016/j.coldregions.2017.07.006.

Li, H. 2009. “Experimental and numerical study of sonic wave propagation in freezing sand and silt.” University of Alaska Fairbanks. https://scholarworks.alaska.edu/handle/11122/8987.

Li, K., Q. Li, and C. Liu. 2022. “Impacts of water content and temperature on the unconfined compressive strength and pore characteristics of frozen saline soils.” KSCE Journal of Civil Engineering, 26 (4): 1652–1661. https://doi.org/10.1007/s12205-022-1037-x.

Li, Z., J. Chen, and M. Sugimoto. 2020. “Pulsed NMR measurements of unfrozen water content in partially frozen soil.” Journal of Cold Regions Engineering, 34 (3): 04020013. American Society of Civil Engineers. https://doi.org/10.1061/(ASCE)CR.1943-5495.0000220.

Liu, B., L. He, C. Li, Y. Han, Y. Sun, Q. Han, and J. Zeng. 2024. “Study on electrical properties of saline frozen soil and influence mechanism of unfrozen water content.” Cold Regions Science and Technology, 104146. https://doi.org/10.1016/j.coldregions.2024.104146.

Liu, J., P. Yang, and Z. (Joey) Yang. 2020. “Electrical properties of frozen saline clay and their relationship with unfrozen water content.” Cold Regions Science and Technology, 178: 103127. https://doi.org/10.1016/j.coldregions.2020.103127.

Ma, H., Y. Cui, J. Zhang, S. Li, and S. Xu. 2023. “Effects of transient temperature gradient on frost heave of saturated silty clay in an open system.” Research in Cold and Arid Regions, 15 (6): 268–277. https://doi.org/10.1016/j.rcar.2023.11.005.

Maas, A. L., A. Y. Hannun, and A. Y. Ng. 2013. “Rectiﬁer nonlinearities improve neural network acoustic models.” In: Proc Icml.

Meng, X., J. Zhou, C. Wei, K. Zhang, Z. Shen, and Z. Yang. 2020. “Effects of salinity on soil freezing temperature and unfrozen water content.” Rock and Soil Mechanics, 41 (3): 952–960. https://doi.org/10.16285/j.rsm.2019.0617.

Ming, F., D. Q. Li, and L. Chen. 2020. “Electrical resistivity of freezing clay: Experimental study and theoretical model.” Journal of Geophysical Research: Earth Surface, 125 (2): e2019JF005267. https://doi.org/10.1029/2019JF005267.

Nakano, Y., A. Tice, J. Oliphant, and T. Jenkins. 1982. “Transport of water in frozen soil: I: Experimental determination of soil-water diffusivity under isothermal conditions.” Advances in Water Resources, 5 (4): 221–226. https://doi.org/10.1016/0309-1708(82)90004-5.

Patterson, D. E., and M. W. Smith. 1985. “Unfrozen water content in saline soils: results using time-domain reflectometry.” Canadian Geotechnical Journal, 22 (1): 95–101. https://doi.org/10.1139/t85-009.

Penner, E. 1970. “Thermal conductivity of frozen soils.” Canadian Journal of Earth Sciences, 7 (3): 982–987. https://doi.org/10.1139/e70-091.

Petersen, L. W., P. Moldrup, O. H. Jacobsen, and D. E. Rolston. 1996. “Relations between specific surface area and soil physical and chemical properties.” Soil Science, 161 (1): 9. https://doi.org/10.1097/00010694-199601000-00003.

Ren, J., and S. K. Vanapalli. 2019. “Comparison of soil-freezing and soil-water characteristic curves of two canadian soils.” Vadose Zone Journal, 18 (1): 180185. https://doi.org/10.2136/vzj2018.10.0185.

Ren, J., and S. K. Vanapalli. 2020. “Effect of freeze–thaw cycling on the soil-freezing characteristic curve of five Canadian soils.” Vadose Zone Journal, 19 (1): e20039. https://doi.org/10.1002/vzj2.20039.

Smerdon, B. D., and C. A. Mendoza. 2010. “Hysteretic freezing characteristics of riparian peatlands in the Western Boreal Forest of Canada.” Hydrological Processes, 24 (8): 1027–1038. https://doi.org/10.1002/hyp.7544.

Smith, M. W., and A. R. Tice. 1988. “Comparison of NMR and TDR methods.”

Suzuki, S. 2004. “Dependence of unfrozen water content in unsaturated frozen clay soil on initial soil moisture content.” Soil Science and Plant Nutrition, 50 (4): 603–606. Taylor & Francis. https://doi.org/10.1080/00380768.2004.10408518.

Tang, L., K. Wang, L. Jin, G. Yang, H. Jia, and A. Taoum. 2018. “A resistivity model for testing unfrozen water content of frozen soil.” Cold Regions Science and Technology, 153: 55–63. https://doi.org/10.1016/j.coldregions.2018.05.003.

Tang, L., X. Wang, F. Lan, P. Qiu, and L. Jin. 2020. “Measuring the content of unfrozen water in frozen soil based on resistivity.” International Journal of Electrochemical Science, 15 (9): 9459–9472. https://doi.org/10.20964/2020.09.57.

Teng, J., J. Kou, X. Yan, S. Zhang, and D. Sheng. 2020. “Parameterization of soil freezing characteristic curve for unsaturated soils.” Cold Regions Science and Technology, 170: 102928. https://doi.org/10.1016/j.coldregions.2019.102928.

Tice, A. R., P. B. Black, and R. L. Berg. 1988. “Unfrozen water contents of undisturbed and remolded alaskan silt as determined by nuclear magnetic resonance.”

Wang, M., X. Li, and X. Xu. 2021. “An implicit Heat-Pulse-Probe method for measuring the soil ice content.” Applied Thermal Engineering, 196: 117186. https://doi.org/10.1016/j.applthermaleng.2021.117186.

Wang, Z., M. Xiao, M. Bray, D. Nicolsky, and X. Ji. 2026. “Creep deformation and long-term strength of ice-rich permafrost in Northern Alaska.” Journal of Geotechnical and Geoenvironmental Engineering, 152 (1): 04025161. American Society of Civil Engineers. https://doi.org/10.1061/JGGEFK.GTENG-13324.

Watanabe, K., and M. Mizoguchi. 2002. “Amount of unfrozen water in frozen porous media saturated with solution.” Cold Regions Science and Technology, 34 (2): 103–110. https://doi.org/10.1016/S0165-232X(01)00063-5.

Watanabe, K., and Y. Osada. 2017. “Simultaneous measurement of unfrozen water content and hydraulic conductivity of partially frozen soil near 0   °C.” Cold Regions Science and Technology, 142: 79–84. https://doi.org/10.1016/j.coldregions.2017.08.002.

Watanabe, K., and T. Wake. 2009. “Measurement of unfrozen water content and relative permittivity of frozen unsaturated soil using NMR and TDR.” Cold Regions Science and Technology, 59 (1): 34–41. https://doi.org/10.1016/j.coldregions.2009.05.011.

Wen, Z., W. Ma, W. Feng, Y. Deng, D. Wang, Z. Fan, and C. Zhou. 2012. “Experimental study on unfrozen water content and soil matric potential of Qinghai-Tibetan silty clay.” Environmental Earth Sciences, 66 (5): 1467–1476. https://doi.org/10.1007/s12665-011-1386-0.

Williams, P. J. 1964. “Unfrozen water content of frozen and soil moisture suction.” https://doi.org/10.1680/geot.1964.14.3.231.

Wu, B., H.-H. Zhu, D. Cao, L. Xu, and B. Shi. 2021. “Feasibility study on ice content measurement of frozen soil using actively heated FBG sensors.” Cold Regions Science and Technology, 189: 103332. https://doi.org/10.1016/j.coldregions.2021.103332.

Wu, M., X. Tan, J. Huang, J. Wu, and P.-E. Jansson. 2015. “Solute and water effects on soil freezing characteristics based on laboratory experiments.” Cold Regions Science and Technology, 115: 22–29. https://doi.org/10.1016/j.coldregions.2015.03.007.

Xiaozu, X., J. L. Oliphant, and A. R. Tice. n.d. “Factors affecting water migration in frozen soils.”

Xin, Q., Y. Su, Y. Cao, X. She, Z. Wang, X. Ling, and T. Yang. 2023. “Experimental and modeling investigation of freezing characteristic curve of silty clay using TDR.” Cold Regions Science and Technology, 205: 103715. https://doi.org/10.1016/j.coldregions.2022.103715.

Yoon, J., J. Jordon, and M. Schaar. 2018. “GAIN: Missing data imputation using generative adversarial nets.” In: Proc. 35th Int. Conf. Mach. Learn., 5689–5698. PMLR.

Yoshikawa, K., and P. P. Overduin. 2005. “Comparing unfrozen water content measurements of frozen soil using recently developed commercial sensors.” Cold Regions Science and Technology, 42 (3): 250–256. https://doi.org/10.1016/j.coldregions.2005.03.001.

Yukselen-Aksoy, Y., and A. Kaya. 2010. “Method dependency of relationships between specific surface area and soil physicochemical properties.” Applied Clay Science, 50 (2): 182–190. https://doi.org/10.1016/j.clay.2010.07.020.

Zhang, M., X. Zhang, J. Lu, W. Pei, and C. Wang. 2019. “Analysis of volumetric unfrozen water contents in freezing soils.” Experimental Heat Transfer, 32 (5): 426–438. Taylor & Francis. https://doi.org/10.1080/08916152.2018.1535528.